King Saud University						460 PGE

College of Engineering 					Date 1–6-1428 H

Petroleum & Natural Gas  Engineering Department		Exam Time : 3 hrs

- The following information on a water-drive gas reservoir is given:

B.V. = 100000 acre-ft	Gas gravity = 0.6	Porosity = 15%

Swi =  15%			T = 140 oF		Pi = 3500psi

Zi = 0.823.

Reservoir pressure has decline to 3000 psi (Z = 0.79) while producing 30 MMM scf and no water production. Calculate cumulative water influx.

------------------------------------------------------------------------------------------------------------

- A) Find the oil lost due to shrinking gas cap in oil reservoir?

B) The initial reservoir pressure is 2900 psi. The bubble point pressure is 2000 psi.

N  =10 MM STB, 	Swc = 16%		Porosity = 13%

Cw = 3.3 *10^-6 psi			Cf  = 4.2 *10^-6 psi

| P, Psi | o, Bbl/STB | t Bbl/STB | Rs, Scf/STB | g, Bbl/Scf | o/g |
| --- | --- | --- | --- | --- | --- |
| 2900 | 1.43 | 1.43 | ? | - | - |
| 2000 | 1.48 | 1.48 | 1350 | - | - |
| 1700 | ? | 1.56 | 1280 | 0.001518 | 38.3 |

The Relative permeability curves are below.

Predict the following items at pressure equal to1700 Psi:

- Cumulative oil production,

- Cumulative gas production,

- Oil recovery,

- Produced gas oil ratio.

- A)  Mention the water influx models commonly used in the petroleum industry?

B) Calculate water influx at the end of 6, 12, 18, and 24 months into a circular reservoir with an aquifer of infinite extent. The initial and the current reservoir pressures are 2500 and 2490 psi, respectively. The reservoir-aquifer system has the following properties.

|  | Reservoir | Aquifer |
| --- | --- | --- |
| Radius, ft | 2000 |  |
| H, | 30 | 25 |
| K, md | 50 | 110 |
| Porosity, % | 15 | 22 |
| w, cp | 0.5 | 0.7 |
| cw, psi-1 | 1*10-6 | 0.7*10-6 |
| cf , psi-1 | 2*10-6 | 0.3*10-6 |

The predicted boundary pressure at the end of each specified period is given below:

| Time, months | Boundary pressure, psi |
| --- | --- |
| 0 | 2500 |
| 6 | 2490 |
| 12 | 2472 |
| 18 | 2444 |
| 24 | 2408 |

tD=6.328X10−3ktφμwctre2

B=1.119φctre2hf

When tD    200

W=−4.29881+2.02566∗tdLn(tD)

4    A) Explain this chart?

B) A reservoir is believed to contain initial oil in place of 250 *106 STB and an initial gas cap of 110*109 SCF. The initial pressure at the gas-oil contact is 4000 psi. The PVT properties of the system, cumulative gas and reservoir production pressure have been reported as follows:

| Time | P, psi | o, bbl/stb | Rs, scf/stb | g, bbl/scf | GP, scf |
| --- | --- | --- | --- | --- | --- |
| 1-1-2006 | 4000 | 1.31 | 500 | 6.5*10-4 | 0 |
| 1-1-2007 | 3300 | 1.23 | 340 | 7.5*10-4 | 12*109 |
| 1-1-2008 | 3000 | 1.22 | 315 | 7.6*10-4 | 26*109 |

The uniform initial water saturation is 21% and water and pore volume compressibility are each 3*10-6 psi-1. Production started at on 1-1-2006 and a constant oil production of 55000 STB/day has been maintained. Water injection started at a constant rate of  45000 bbl/d on 1-1-2007. Assume w for injected water is 1.02 bbl/ STB. Estimate the cumulative water influx on 1-1-2008 of the surrounding aquifer.

Best wishes

AG