King Saud University						460 PGE

College of Engineering 					Date 9–5-1428 H

Petroleum & Natural Gas  Engineering Department		Exam Time : 1 hr.

______________________________________________________________________

- The initial reservoir pressure is 2925 psi. The bubble point pressure of 2100 psi at 175o F.

N  =10 MMSTB, 		Swc = 15%		Porosity = 12%

Cw = 3.6 *10^-6 psi	Cf  = 4.9 *10^-6 psi

| P, Psi | o, Bbl/STB | t Bbl/STB | Rs, Scf/STB | g, Bbl/Scf | o/g |
| --- | --- | --- | --- | --- | --- |
| 2925 | 1.43 | 1.43 | 1350 | - | - |
| 2100 | 1.48 | 1.48 | ? | - | - |
| 1800 | 1.47 | 1.56 | 1280 | ? | 38.3 |

The min. Bottom Hole Flowing Pressure = 1000 Psi

The IPR’s for the field at 2925, 2100, and 1800 Psi are below.

The Relative permeability curves are below.

Predict the following items at pressure equal to1800 Psi:

- Cumulative oil production,

- Cumulative gas production,

- Oil flow rate,

- Time,

- Oil recovery,

- Produced gas oil ratio.

Best wishes

AG