1st midterm exam

Mobile Radiography

Rad 1304

Student Name :

……………………………………………

Student No. :

……………………………………………

First Question :

Circle the correct answer :

- The difference between mobile and portable unit is :

  - power out put .

  - ability to transfer the equipment .

  - in x-ray tube .

  - ( a ) and ( b ) are correct .

- Mobile units are classified by tow ways :

  - out put and power supply .

  - out put and machine size .

  - input and ability to transfer .

3. The C-shaped spurted arm rotated to allow :

a. vertical beam .

b. horizontal beam .

c. angles between vertical and horizontal beam .

d. all of the above .

5. The methods of recording the images in C-arm unit are :

a. video-tape , CD and DVD .

b. sorting images during fluoroscopy and transfer them to x-ray                               films  .

c. ( a ) and ( b ) are correct .

d. non of the above .

6. pelvimetry , IVU and pediatric x-ray need :

a. low power mobile .

b. average power mobile .

c. high power mobile .

d. all of the above .

Second Question :

Put ( T ) or ( F ) for the following :

1. A mobile x-ray equipment is an equipment which can be moved from one place to another . (        )

- We can use the mobile x-ray equipments with out-patient . (          )

- As we increase the line voltage of the portable unit , the current following is increased. (          )

- The minimum output of portable unit usually is 80 kvp and 20 mA . (        )

- The war x-ray equipments use a low power mobile . (          )

- The disadvantage of using a capacitor discharge equipment is the limitation in exposure . (          )

- Reduced operation time can be achieved by using the C-arm mobile unit in

theater . (          )

- You can limit the exposure time in C-arm mobile unit by using a pulsed fluoroscopy . (          )

- In C-arm mobile , you use 3 mA for radiography and 30-40 mA for

fluoroscopy . (          )

- The mobile base of C-arm unit carries the control unit and H.T generator . (          )