CHE 201 Tutorial 1

Q1    Using dimensional equation, convert:

- 3 wk to milliseconds.

- 26 miles/h to ft/sec

- 26.7 cm4 /min2g to m4 /day2.kg

Q2    Using the table of conversion factor on the inside front cover, convert

- 760  miles/h to m/s

- 921 kg/m3  to Ibm/ft3

- 5370  kJ/min to hp

Q3    Calculate

- The weight in Ibf of a 25 Ibm object.

- The mass in kg of a 25 N object.

- The weight in dynes of a 10 ton  object.

Q4      Prandtl number Pr is a dimensionless group, it is defined as:

Pr=CPμk

Where

CP = heat capacity of a fluid

   = fluid viscosity

k   = thermal conductivity.

Calculate Pr for a particular fluid, CP = 0.583 J/g C, k = 0.286W/m.C and  = 1936 Ibm / ft h.

Q5      Reynolds number Re  is a dimensionless group, it is defined as:

Re=Dvρμ

Where

D  = pipe diameter

   = fluid viscosity

v   = fluid velocity.

   = fluid density.

Calculate Re for a particular fluid,  = 0.805 g/cm3, D =2.067 in ,  = 0.43 centipoise (1 cP = 0.001kg/m s) and v = 0.48 ft/s.