CHE 201 Tutorial 2

Q1

The following dimensionless equation correlates the values of variables in a system in which solid particles are suspended in a flowing gas.

kgdpyD=2+0.6μρD13dpvρμ12

Both (/ D) and (dp v/) are dimensionless groups. kg is a coefficient that expresses the rate at which a particular species transfers from the gas to the solid particles. The variable values are as follows:

D  =  0.1 cm2/sec

   = 1 x 10-5 N sec/m2

v   = 10 m/sec

   = 1x 10-3 g/cm3

dp  = 5 mm

y   = 0.1 (dimensionless)

what is the estimated value of kg ? (show units)

Q2

A seed crystal of diameter D(mm) is placed in a solution of dissolved salt and new crystals are observed to nucleate (form) at a constant rate r (crystal/min). Experiments run with seed crystals of different sizes show that the rate of nucleation varies with seed crystal diameter as

r(crystal/min)=200D−10D2,    D (mm)

- What are the units of the constants 200 and 10 ?

- Derive the formula for r (crystal/s) in terms of D (inches).

Q3

The density of a fluid is given by the empirical equation

ρ=70.5exp(8.27x10−7P)

Where  = density (Ibm/ft3) and P= pressure (Ibf/in2).

- What are the units of 70.5 and 8.27x10-7?

- Derive the formula for ( g/cm3) as a function of P(N/m2).

- Calculate the density in g/cm3 for a pressure of 9x106 N/m2.

Q4

The specific gravity of gasoline is approximately 0.7

- Determine the mass (kg) of  50 liters of gasoline.

- The mass flow rate existing a refinery tank is 1150 kg/min, estimate the volumetric flow rate in L/sec.