CHE 201 Tutorial 3

Q1

Consider a gas mixture which contains CO=42%, H2=8%, CH4=16% and NH3=34% by mass (xi).

- Calculate the average molecular weight of this mixture.

- Calculate the molar composition (yi) of the gas mixture.

Data: Atomic mass: H=1, C=12, O=16, N=14

Q2

A mixture of gases contains equimolar amounts of ethylene, ethane and propane. Calculate the mass fraction of the components in mixture and average molecular weight.

Q3

A mixture is 10 mol% ethyl alcohol, 75 mol% ethyl acetate (C4H8O2), and 15 mol% acetic acid. Calculate the mass fraction of each compound. What is the average molecular weight of this mixture? What would be the mass (kg) of a sample containing 25 kmol of ethyl acetate?

Q4

A mixture of methanol (CH3OH) and methyl acetate (C3H6O2) contains 15 wt % methanol.

- Determine the g-mole of methanol in 200 kg of the mixture.

- The flow rate of methyl acetate in the mixture is to be 100 Ib-mole/h. What must the mixture flow rate be in Ibm/h?

Q5

Liquid toluene is flowing through a pipe at a rate of 175 m3/h. What are the mass flow rate (kg/min) and the molar flow rate (mol/s and Ibmole/min) of this stream?

Q6

Convert the following temperatures in parts (a) and (b) and temperature interval in parts (c) and (d):

- T = 85ºF to R, C, K

- T = -10 C to K, ºF, R

- T = 85 C to K, ºF, R

- T = 150 R to ºF, C, K

Q7

Perform the following pressure conversion, assuming when necessary that atmospheric pressure is 1 atm. Unless otherwise stated, the given pressures are absolute.

- 2600 mmHg to psi

- 275 ft H2O to kPa

- 3 atm to N/cm2

- 280 cm Hg to dyne/m2

- 20 cm Hg of vacuum to atm (absolute)

- 25 psig to mm Hg (gauge)

- 25 psig to mm Hg (absolute)

- 325 mm Hg to mm Hg gauge

- 35 psi to cm of carbon tetrachloride