King Saud University

College of Engineering

Civil Engineering Department

SE 211

CHAIN SURVEYING

SUMMARY & ABSTRACT:

Chain (Tape) surveying is the simplest form of detail surveying. In this method the lengths of lines marked on the field are measured, while the details are measured by offsets and ties from these lines. This field work will continue for 3 field hours. Every group is expected to submit a detailed report besides the final plan of the area surveyed.

OBJECTIVES:

This field work aimed to train the student on the following process:

- Selection of a frame work (chain or base lines) and control points.

- Direct method of linear measurements (horizontal distance measurement).

- Setting out right angles (offsets).

- Determining the direction of any line in the field with respect to magnetic north "bearing".

- Booking Method.

- Plotting Method.

APPARATUS & INSTRUMENT

- 2 Tapes

- 3 Ranging Rods

- Arrows/Marker paints

- Prism Square

- Prismatic Compass

- Booking Board

PROCEDURE:

- Make a reconnaissance of the area and select a suitable framework (chain lines) and stations based on the criterion given in Note 1.

- Mesure all the chain (based) lines once in each direction using the direct measurement method. See Note 2.

- Measure the offsets/or ties from every necessary point on the detail to the corresponding base line. Use the procedure explained in Note 3.

- You may need some measurements on the details (on the building sides…).

- Measure the bearing of one of the chain lines by using the prismatic compass. See Note 4.

- Record all the information and measurements properly in the booking papers as explained in Note 5.

- Make the necessary calculations and corrections.

- Draw the details in a suitable scale to produce a detailed map of the area. See Note 6.

Note 1: CHAIN LINES & STATIONS SELECTIOS

To locate a suitable stations and chain lines, a reconnaissance of the area should be undertaken by walking around the area required to be surveyed. Any obstacles should be noticed. The selected stations should produce a well formed linked triangles or braced quadrilaterals.

The principles to be considered are:

- Few long lines should be used.

- Avoid any obstacles to ranging or chaining.

- Angles should be > 30o & <120o.

- Make check lines when possible to detect errors when plotting.

- The lines should be closed to the details (Avoid long offsets (>10m) and ties.

After selection of the framework you should draw sketch of the area and mark the stations by wooden pegs or marker paints, and give a number for each station.

NOTE 2: DIRECT DISTANCE MEASUREMENT

For measuring any distance between A and B, especially when it is longer than the tape length, the following ranging is needed: "ranging by eye".

- The ranging rode should be placed at the beginning point A and the end point B. The first man should place the tape zero point at A.

- The second man holding the third raging rod and the tape should move to the direction of B to a point of the end of the tape length. Then he should hold the ranging rod vertically.

- The first man at point A lines up the ranging rod with that on point B. "by sighting as low as possible".

- Then, the second man should straighten the tape and mark this point.

- Both men move ahead repeat the whole steps beginning from the first mark.

- As measurements processes, the second man should record the length.

Note 3: SETTING THE OFFSET

Any point on the detail can be related to the chain line by offset or ties. This is accomplished by measuring all X's and Y's as shown in the figure.

To set out offsets, a prism square can be used. To locate the point at which a 1 from any point on the feature would meet the chain line (say AB) you can follow the following steps:

- One man should hold the ranging pole at the given point, while the other pole is placed at any point on the chain line AB.

- The observer holds the instrument and walk along the line AB until he see both poles coincide in each other. Then the distance to this point along the chain line can be measured beside the length of the perpendicular.

Another method can be used by holding the zero point of the tape at the given point and swinging tape over the chain line and mark the point on the chain line at minimum reading.

NOTE 4: MEASURING THE BEARING OF A LINE

To measure the angle that any line makes with the magnetic direction, you can use a prismatic compass. The procedure is as follow:

- Place the ranging pole vertically at point B.

- Place the prismatic compass over its tripod at point A and level it using the bubble and screws.

- Rotate the compass until it is directed to the pole and read the angle.

- Repeat the whole procedure for point B.

NOTE 5: BOOKING

The field book should be neat and consistent:

- Each chain line is represented by double line drawn through the corner of the page.

- Entries start at the bottom of the page.

- Detail that is on the right-hand side of the line is booked on the right-hand side of the page and voice versa.

- The lengths from the beginning of the line are written inside the double lines while the offset lengths outside.

NOTE 6: PLOTTING

A.  Plot the framework of chain lines by:

- Draw the longest line according to its bearing.

- Build up the other chain lines by using beam compasses.

- Draw the check lines, and if there are any errors, check the drawn lines in the incorrect triangles. (You may need to measure them in the field).

B. Draw the detail for each chain line based on offsets and ties information. Then connect these points to get the details.