King Fahd University of 	Petroleum and Minerals

Department of Electrical Engineering

EE 201 Electric Circuits I,

Major Exam I

Monday, 19 March 2007

6:30 pm – 8:00 pm

Name:

ID:

Section:

Instructor: Dr. Yahya Al-Harthi

| Problem | Score | Out of |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| Total |  |  |

Good luck!

- Find the equivalent resistance Rab for the following circuit.

- Ali has four devices (A, B, C, and D) which should sit on the board and be connected to the voltage source to function. These devices have the following specifications:

- Internal resistors:RA=RB=RC=RD

- Needed Voltages:  VA=2VB=VC=2VD=VS

- Draw the connection of these devices (assume the devices are square boxes).

- If VS=10VandRB=2Ω, what is the value of the current going through device B?

- For the circuit shown below:

  - Use mesh-current analysis to write the equations of the currents given the speakers' internal resistors are: R1,R2,R3, and R4.

  - IfR1=R2=R3=R4=2Ω, I2=1 A, and Vs=2 V find the values of the currents.

  - If we want to replace the voltage source with a current source while still keeping the values of the currents in part (b), what should be the value of this current source?