Maintenance Workload Forecasting

Example 3.1:

A company wants to predict the maintenance workload of its mechanics for the first period of the next year. Before the company does that, it has been noted that in the first period of the next year, the company manufacturing plant will pass over circumstances similar to those that it faced in the first six months of the last year. What is the mechanics maintenance workload for the first period of the next year if the actual maintenance workloads for these mechanics in the previous periods are given in table 1?

| Periods | Actual Maintenance Workload (hours) | Periods | Maintenance Workload (hours) |
| --- | --- | --- | --- |
| 1 | 120 | 7 | 111 |
| 2 | 100 | 8 | 130 |
| 3 | 110 | 9 | 110 |
| 4 | 90 | 10 | 125 |
| 5 | 95 | 11 | 120 |
| 6 | 105 | 12 | 100 |

Example 3.2:

A manufacturing plant wants to predict the maintenance work load of its electricians for the next planning period in one of its manufacturing area. The engineers in the maintenance department found that during the last six periods, a percentage of the actual maintenance workload performed during each one of these periods represents a part of the forecasted maintenance work load for the electricians in the next planning period. In particular, they found the following:

1. 22% of the actual maintenance work load in the first period of the last six periods represents the first part of the forecasted maintenance work load of the electricians.

2. 18% of the actual maintenance work load in the second period of the last six periods represents the second part of the forecasted maintenance work load of the electricians.

3. 15% of the actual maintenance workload in the other remaining periods represents the remaining parts of the forecasted maintenance work load of the electricians

The actual maintenance workloads in the previous periods are given in table 2.

| Periods | Actual Maintenance Workload (hours) |
| --- | --- |
| 1 | 150 |
| 2 | 130 |
| 3 | 160 |
| 4 | 100 |
| 5 | 130 |
| 6 | 110 |

Example 3.3:

Calculate the next planning period maintenance workload for a manufacturing area in a plant that has 12 machines, if the recent actual maintenance workloads reflect only 40% of the amount of this manufacturing area next period maintenance workload. The actual maintenance workloads in the previous periods are given in table 3.

| Periods | Actual Maintenance Workload (hours) |
| --- | --- |
| 1 | 30 |
| 2 | 37 |
| 3 | 25 |
| 4 | 35 |
| 5 | 20 |

Example 3.4:

A company wants to predict the maintenance workload for the next planning period for one of its manufacturing area. The area has six machines in total. It started its production in the first two periods by using two of these six machines. After that, all of the six machines were operated to cover the production demand. These six machines will still be operated in the next planning period. The actual maintenance workloads in the previous periods for the same area are given in table 4.  Find the next planning period maintenance workload.

| Periods | Actual Maintenance Workload (hours) | Periods | Actual Maintenance Workload (hours) |
| --- | --- | --- | --- |
| 1 | 60 | 7 | 327 |
| 2 | 50 | 8 | 320 |
| 3 | 360 | 9 | 316 |
| 4 | 345 | 10 | 312 |
| 5 | 336 | 11 | 305 |
| 6 | 330 | 12 | 300 |