Chapter 19

Statistical Process Control and Quality Management

True/False

1.  Dr. W. Edwards Deming proposed a philosophy of managing quality based on list of 14 points.

Answer: True        Difficulty: Easy        Goal: 1        AACSB: REF

2.  The Malcolm Baldridge National Quality Award recognizes American companies that demonstrate excellent quality management.

Answer: True        Difficulty: Easy        Goal: 1

3.  The Malcolm Baldridge National Quality Award recognizes American companies that demonstrate excellent application of statistical methods in decision making.

Answer: False        Difficulty: Easy        Goal: 1

4.  In quality management, "Six Sigma" refers to plus or minus three standard deviations.

Answer: False        Difficulty: Easy        Goal: 1        AACSB: AS

5.  In quality management, "Six Sigma" refers to a company-wide program to improve quality and overall performance.

Answer: True        Difficulty: Easy        Goal: 1

6.  A fishbone diagram is used to determine if a process is out of control

Answer: False        Difficulty: Easy        Goal: 4

7.  Dr. Walter A. Shewhart introduced the concepts of "controlling" the quality rather than inspecting quality into the part.

Answer: True        Difficulty: Easy        Goal: 1

8.  There are two general types of causes of variations in manufacturing and service processes: chance and assignable causes.

Answer: True        Difficulty: Easy        Goal: 2

9.  Assignable causes are usually large in number and random in nature, and they cannot be entirely eliminated.

Answer: False        Difficulty: Easy        Goal: 2

10.  Chance causes of variation are usually few in number and not random in nature, and they can be readily identified, and reduced or eliminated.

Answer: False        Difficulty: Easy        Goal: 2

11.  Assignable causes can generally be corrected easily, while chance causes cannot usually be corrected or stabilized economically.

Answer: True        Difficulty: Easy        Goal: 2        AACSB: AS

12.  The purpose of a statistical quality control chart is to identify when assignable causes of variation have entered the production system so that the cause may be identified and corrected.

Answer: True        Difficulty: Easy        Goal: 2        AACSB: AS

13.  Pareto charts can be used to identify and diagnose chance causes of variation in a manufacturing or service process.

Answer: False        Difficulty: Easy        Goal: 2

14.  Pareto charts can be used to identify and diagnose assignable causes of variation in a manufacturing or service process.

Answer: True        Difficulty: Easy        Goal: 2

15.  Pareto charts are based on the 80-20 rule, which says, 80% of the problem is caused by 20% of the factors.

Answer: True        Difficulty: Easy        Goal: 2        AACSB: AS

16.  Pareto charts are based on the 80-20 rule, which says, 80% of the factors cause 20% of the problem.

Answer: False        Difficulty: Easy        Goal: 2

17.  Quality control charts indicate whether a production process is in control or out of control.

Answer: True        Difficulty: Easy        Goal: 2        AACSB: AS

18.  Control charts can be developed for both variables and attributes.

Answer: True        Difficulty: Easy        Goal: 2

19.  Mean charts are designed to determine whether a process is in or out of control based on sample means.

Answer: True        Difficulty: Easy        Goal: 5

20.  Summing the means of the samples and dividing that sum by the number of sample means can compute the mean of an X bar chart.

Answer: True        Difficulty: Easy        Goal: 5

21.  The purpose of a mean chart is to portray the fluctuations in the sample means.

Answer: True        Difficulty: Easy        Goal: 5

22.  The upper control limit (UCL) and the lower control limit (LCL) of a chart report the limits of the variation expected by chance.

Answer: True        Difficulty: Medium        Goal: 5        AACSB: AS

23.  Mean charts use sample ranges to determine if the process is in or out of control.

Answer: False        Difficulty: Easy        Goal: 5

24.  For a range chart with control limits set at ± 3ó, about 997 sample ranges out of 1,000 will fall between the upper and lower control limits.

Answer: True        Difficulty: Medium        Goal: 5

25.  A range chart shows variation in the ranges of the samples.

Answer: True        Difficulty: Easy        Goal: 5        AACSB: REF

26.  If one sample range should fall outside the control limits, it is very likely that some assignable cause affected the production.

Answer: True        Difficulty: Easy        Goal: 5

27.  If one sample range should fall outside the control limits, it is very unlikely that some chance event affected the production.

Answer: True        Difficulty: Easy        Goal: 5

28.  Percent defective charts, which are also known as P charts or p bar charts, show the percent of the production that is or is not acceptable.

Answer: True        Difficulty: Easy        Goal: 6

29.  A c bar chart shows the percent of the production that is defective.

Answer: False        Difficulty: Easy        Goal: 6

30.  The following are examples of attributes: A weld has a crack in it or it doesn't; a relay works or it doesn't; a radiator leaks or it doesn't; the lock on a car door works or it doesn't; a tire fits on the rim or it doesn't.

Answer: True        Difficulty: Easy        Goal: 2

31.  The following are examples of attributes: the width of a cracked weld, the length of time that a relay works; the amount of water in a radiator, the diameter of a tire.

Answer: False        Difficulty: Easy        Goal: 2

32.  The following are examples of variables: the width of a cracked weld, the length of time that a relay works; the amount of water in a radiator, the diameter of a tire.

Answer: True        Difficulty: Easy        Goal: 6

33.  The purpose of a c bar chart is to show how many defects appear in a unit of production.

Answer: True        Difficulty: Easy        Goal: 6        AACSB: AS

34.  A value of 4.0 is recorded on the centerline of a c bar chart.  This indicates that the lowest number of defects per unit is 4.0.

Answer: False        Difficulty: Easy        Goal: 6

35.  Chance causes of variation cannot be entirely eliminated.

Answer: True        Difficulty: Easy        Goal: 2

36.  In acceptance sampling, usually 100% of the incoming lot is checked.

Answer: False        Difficulty: Easy        Goal: 7

37.  In acceptance sampling, if the number of defects in a sample is less or equal to the acceptance number, then the incoming lot is accepted.

Answer: True        Difficulty: Easy        Goal: 7

Multiple Choice

38.  Samples of 200 parts were taken every day and the number of defectives was counted. What are the upper and lower limits for the percent defective chart?

A)  0.02 and 0

B)  0.2970 and 0

C)  0.0297 and 0

D)  0.0497 and 0

Answer: D        Difficulty: Hard        Goal: 6

39.  A subassembly is inspected and the number of defects recorded.  A new group of assemblers began work Monday morning.  The number of defects per subassembly for the first 10 they produced was: 3, 2, 0, 5, 4, 6, 0, 7, 7, and 6. What are the upper and lower limits for the c bar chart?

A)  15 and 0

B)  10 and 0

C)  1.12 and 0

D)  6 and –2

Answer: B        Difficulty: Hard        Goal: 6

40.  What is usually developed to evaluate a sampling plan?

A)  Frequency polygon

B)  Scatter diagram

C)  Operating characteristic curve

D)  Simple bar chart

Answer: C        Difficulty: Medium        Goal: 7

41.  Which probability distribution is used to develop a percent defective chart?

A)  Normal distribution

B)  Chi-square distribution

C)  Mann-Whitney test

D)  Binomial distribution

Answer: D        Difficulty: Medium        Goal: 6        AACSB: AS

42.  Which probability distribution is used to develop a control chart for sample means?

A)  Normal distribution

B)  Chi-square distribution

C)  Mann-Whitney test

D)  Binomial distribution

Answer: A        Difficulty: Medium        Goal: 8

43.  Which probability distribution is used to develop an operating characteristic curve?

A)  Normal distribution

B)  Chi-square distribution

C)  Mann-Whitney test

D)  Binomial distribution

Answer: D        Difficulty: Medium        Goal: 8

44.  If the weather forecast says that there will be a 20% chance of rain, what type of variation would rain be an example?

A)  Chance

B)  External

C)  Assignable

D)  Standard variation

Answer: A        Difficulty: Medium        Goal: 2        AACSB: REF

45.  If the weather forecast says that there will be a 100% chance of rain, what type of variation would rain be an example?

A)  Chance

B)  External

C)  Assignable

D)  Standard variation

Answer: C        Difficulty: Medium        Goal: 2

46.  A sampling plan states that if 20 incoming transistors are checked and 2 or less defects are found that the lot is accepted.  If an incoming lot is 10 percent defective, what is the probability of accepting the lot of transistors?

A)  0

B)  1

C)  0.037

D)  0.667

E)  0.333

Answer: D        Difficulty: Hard        Goal: 8

47.  A sampling plan states that if 20 incoming bolts are checked and 2 or less defective bolts are discovered the lot will be rejected.  If an incoming lot is 10 percent defective, what is the probability of rejecting the lot?

A)  0

B)  1

C)  0.333

D)  0.667

Answer: C        Difficulty: Hard        Goal: 8        AACSB: AS

48.  Cappelli Inc. designs and manufactures women's apparel using material from various mills.  Their acceptance sampling plan states that 20 two inch squares of the incoming material must be carefully checked.  If 3 or less squares reveal imperfections, the lot is accepted.  What is the probability that an incoming lot from Blufton Mills that contains 40 percent imperfect cloth will be accepted?

A)  0

B)  0.239

C)  0.015

D)  0.0024

Answer: C        Difficulty: Hard        Goal: 8

49.  What is the process that insures that a company is producing a quality product or service?

A)  SPC

B)  Pareto analysis

C)  Fishbone chart analysis

D)  Diagnostic chart

Answer: A        Difficulty: Easy        Goal: 5

50.  What chart helps to identify the relatively few factors that impact the performance of a manufacturing or service process?

A)  SPC

B)  Pareto analysis

C)  Fishbone chart analysis

D)  Diagnostic chart

Answer: B        Difficulty: Easy        Goal: 3

51.  What process investigates the potential causes of an outcome or effect such as poor quality?

A)  SPC

B)  Pareto analysis

C)  Fishbone chart analysis

D)  Diagnostic chart

Answer: C        Difficulty: Easy        Goal: 4        AACSB: REF

52.  A fishbone diagram classifies potential causes into categories.  What are they?

A)  Chance and assignable variation

B)  Methods, materials, equipment, and personnel

C)  Percent defective and percent non-defective

D)  Causes and effects

Answer: B        Difficulty: Easy        Goal: 4

53.  What type of variation can be reduced, or even eliminated, and is usually nonrandom in nature?

A)  Chance

B)  External

C)  Assignable

D)  Co-variation

Answer: C        Difficulty: Medium        Goal: 2

54.  Equipment failure would be an example of ____________ variation.

A)  chance

B)  external

C)  assignable

D)  standard variation

Answer: C        Difficulty: Medium        Goal: 2

55.  The technique that tallies types of defects and their frequency is called

A)  range charts.

B)  Pareto analysis.

C)  control charts.

D)  fishbone diagram.

Answer: B        Difficulty: Medium        Goal: 3        AACSB: CA

56.  A chart used to show the results of a Pareto analysis is a

A)  Range chart.

B)  Bar chart.

C)  Control chart.

D)  Fishbone diagram.

Answer: B        Difficulty: Medium        Goal: 3

57.  The cause-and-effect chart is an example of a

A)  range chart.

B)  Pareto chart.

C)  control chart.

D)  fishbone diagram.

Answer: D        Difficulty: Medium        Goal: 4

58.  In a control chart, expected variation is defined by the

A)  standard deviation.

B)  standard error of the distribution.

C)  only upper control limits.

D)  upper and lower control limits.

Answer: D        Difficulty: Medium        Goal: 5        AACSB: REF

59.  Control charts are used to determine if a process is

A)  in or out of control.

B)  measurable.

C)  fixable.

D)  testable.

Answer: A        Difficulty: Medium        Goal: 5

60.  The Quality Assurance Department selected a sample of 12 printed circuit boards and tested them.  The number of defects in each circuit board was 3, 3, 0, 5, 1, 1, 5, 6, 6, 2, 0, and 1.  What kind of control chart should be constructed to monitor the process?

A)  Mean chart

B)  Range chart

C)  Percent defective chart

D)  C-bar chart

Answer: D        Difficulty: Easy        Goal: 6        AACSB: AS

61.  The Quality Assurance Department selected 12 samples of 100 printed circuit boards and tested them.  The number of defective printed circuit boards in each sample was 3, 3, 0, 5, 1, 1, 5, 6, 6, 2, 0, and 1.  What kind of control chart should be constructed to monitor the process?

A)  Mean chart

B)  Range chart

C)  Percent defective chart

D)  C-bar chart

Answer: C        Difficulty: Easy        Goal: 6

62.  The upper and lower control limits are usually set at

A)  the mean.

B)  above the mean.

C)  3 standard deviations from the mean.

D)  2 standard deviations from the mean.

Answer: C        Difficulty: Medium        Goal: 5

63.  If the upper and lower control limits are set at 3 standard deviations from the mean, we would expect that

A)  99.7% of the sample means are within the control limits

B)  95% of the sample means are within the control limits

C)  about 68% of the sample means are within the control limits

D)  about 5% of the sample means are outside the control limits

Answer: A        Difficulty: Medium        Goal: 5

Fill-in-the-Blank

64.  Which chart portrays the number of defects per unit of production? _____________

Answer: c bar         Difficulty: Medium        Goal: 6

65.  In an acceptance sampling plan, what happens to the incoming lot if the number of defective items in the sample is less than or equal to the acceptance number? __________________

Answer: The lot is accepted.         Difficulty: Medium        Goal: 6

66.  In acceptance sampling, it is possible that an incoming lot of computer chips contains more defective chips than it should, but it is accepted.  In acceptance sampling, what is this probability call?   _______

Answer: consumers risk         Difficulty: Medium        Goal: 7

67.  If the binomial distribution is used to develop an operating characteristic curve for an acceptance sampling plan, then it is imperative that the trials be ________________

Answer: independent         Difficulty: Medium        Goal: 7

68.  If the binomial distribution is used to develop an operating characteristic curve for an acceptance sampling plan, how many possible outcomes can there be?  _______

Answer: two         Difficulty: Medium        Goal: 1

69.  In what decade were the concepts of statistical quality control developed?________

Answer: 1920's         Difficulty: Medium        Goal: 1

70.  One of the most important concepts of statistical process control is that no two products are ________________

Answer: identical         Difficulty: Medium        Goal: 1

71.  What does a statistical quality control chart identify? ____________

Answer: assignable variation         Difficulty: Medium        Goal: 2

72.  Acceptance sampling and control charts may be based on measurements with either ______________ or ____________________.

Answer: variables or attributes         Difficulty: Medium        Goal: 1

73.  What is the purpose of the mean chart? _______________________________

Answer: To show the variation in sample means         Difficulty: Medium        Goal: 5

74.  How is the mean of the sample means designated? _______

Answer: X double bar         Difficulty: Medium        Goal: 5

75.  A range chart shows variation of _____________ .

Answer: sample ranges         Difficulty: Medium        Goal: 5

76.  If the sample ranges fall randomly within the upper and lower control limits, then it can be assumed that production is ____________ .

Answer: in control         Difficulty: Medium        Goal: 5

77.  A p-bar chart graphs the portion of production that is not acceptable based on random samples.  What is another name for the p-bar chart? ______________

Answer: percent defective chart         Difficulty: Medium        Goal: 6

78.  What chart portrays the number of defects per unit? ____________

Answer: c-bar         Difficulty: Medium        Goal: 6

79.  What is the "in control" region of a control chart called? ____________________

Answer: the acceptance region         Difficulty: Medium        Goal: 1

80.  What causes of variation cannot be entirely eliminated? _____________

Answer: chance         Difficulty: Medium        Goal: 1

81.  What variation is due to slight changes in materials or process conditions and is random in nature? ________________

Answer: chance         Difficulty: Medium        Goal: 2

Multiple Choice

Use the following to answer questions 82-98:

A new machine is set or calibrated to fill tubes of toothpaste at precisely 6.75 ounces.  Then, once the machine is running, a sample of 6 tubes is taken every three hours and weighed with the following results:

82.  If the mean of the ranges is 0.08, what is the value of the upper control limit (UCL) for the range?

A)  0.000

B)  0.160

C)  0.169

D)  0.203

Answer: B        Difficulty: Medium        Goal: 5        AACSB: AS

83.  If the mean of the ranges is 0.08, what is the value for the lower control limit (LCL) of the range?

A)  0.00

B)  0.080

C)  0.186

D)  0.203

Answer: A        Difficulty: Medium        Goal: 5

84.  What is X bar for 7 am?

A)  6.72

B)  6.74

C)  6.78

D)  6.80

Answer: B        Difficulty: Medium        Goal: 5

85.  What is X bar for 10 am?

A)  6.73

B)  6.74

C)  6.75

D)  6.76

Answer: A        Difficulty: Medium        Goal: 5

86.  What is X bar for 1 pm?

A)  6.73

B)  6.74

C)  6.75

D)  6.71

Answer: D        Difficulty: Medium        Goal: 5

87.  What is X bar for 4 pm?

A)  6.73

B)  6.74

C)  6.75

D)  6.80

Answer: C        Difficulty: Medium        Goal: 5

88.  What is X bar for 7 pm?

A)  6.71

B)  6.72

C)  6.74

D)  6.76

Answer: D        Difficulty: Medium        Goal: 5

89.  What is the value of A2, the control limit factor for averages?

A)  0

B)  1

C)  0.577

D)  0.483

Answer: D        Difficulty: Medium        Goal: 5

90.  What is the range of the samples at 7 am?

A)  0.05

B)  0.06

C)  0.11

D)  0.12

Answer: C        Difficulty: Medium        Goal: 5

91.  What is the range of the samples at 10 am?

A)  0.05

B)  0.06

C)  0.11

D)  0.12

Answer: A        Difficulty: Medium        Goal: 5

92.  What is the range of the samples at 1 pm?

A)  0.05

B)  0.06

C)  0.11

D)  0.12

Answer: D        Difficulty: Medium        Goal: 5

93.  What is the range of the samples at 4 pm?

A)  0.05

B)  0.06

C)  0.11

D)  0.12

Answer: B        Difficulty: Medium        Goal: 5

94.  What is the range of the samples at 7 pm?

A)  0.05

B)  0.06

C)  0.11

D)  0.12

Answer: B        Difficulty: Medium        Goal: 5

95.  What is the value of the upper control factor of the range?

A)  0

B)  2.115

C)  2.004

D)  3.0

Answer: C        Difficulty: Medium        Goal: 5

96.  What is the value of the lower control factor of the range?

A)  0

B)  2.115

C)  2.004

D)  3.0

Answer: A        Difficulty: Medium        Goal: 5

97.  If the mean of the sample means is 6.738 and R bar is 0.08, what is the upper control limit (UCL)?

A)  6.738

B)  6.999

C)  6.777

D)  7.221

Answer: C        Difficulty: Medium        Goal: 5

98.  If the mean of the sample means is 6.738 and R bar is 0.08, what is the lower control limit?

A)  6.738

B)  6.669

C)  6.699

D)  6.255

Answer: C        Difficulty: Medium        Goal: 5

Use the following to answer questions 99-100:

Every half hour, the quality control inspector checks a process by measuring four pieces and records the outside diameters of each of them, as shown below.

99.  What are the upper and lower control limits of the mean chart?

A)  5.916. and 0

B)  5.916 and 0.084

C)  0.729 and 0

D)  4 and 0

Answer: B        Difficulty: Medium        Goal: 5

100.  What are the upper and lower limits of the range chart?

A)  9.128 and 0

B)  2.282 and 4

C)  2.282 and 0

D)  100 and 0

Answer: A        Difficulty: Medium        Goal: 5

Use the following to answer questions 101-103:

A new machine used in the production of motor mount bolts was put into operation.  Five samples of size 100 were randomly selected and the number of defectives in each sample was noted.

101.  What is the average percent defective?

A)  2.0%

B)  2.3%

C)  2.6%

D)  2.9%

Answer: C        Difficulty: Medium        Goal: 6   AACSB: AS

102.  What is the upper control limit (UCL)?

A)  0.067

B)  0.074

C)  0.081

D)  1.000

Answer: B        Difficulty: Medium        Goal: 6

103.  What is the lower control limit (LCL)?

A)  0

B)  –0.022

C)  0.048

D)  0.160

Answer: A        Difficulty: Medium        Goal: 6

Use the following to answer questions 104-107:

Subgroups of 4 items each are taken from a manufacturing process at regular intervals and a certain quality characteristic is measured.  After 25 subgroups, .

104.  What is the value of the means?

A)  15,350

B)  614

C)  411.4

D)  3837.5

Answer: B        Difficulty: Medium

105.  What is the value for R bar?

A)  411.4

B)  614

C)  16.46

D)  102.88

Answer: C        Difficulty: Medium        Goal: 5

Fill-in-the-Blank

106.  What is the upper control limit (UCL) for the mean? _____

Answer: 625.996         Difficulty: Medium        Goal: 5

107.  What is the lower control limit (LCL) for the mean? _______

Answer: 602.004         Difficulty: Medium        Goal: 5

Essay

108.  The manager of a restaurant surveys customer satisfaction as they leave the restaurant.  Is this an example of acceptance sampling or statistical process control?  Explain why.

Answer:  This is an example of acceptance sampling.  The measurement of quality happens after the service has been completed.       Difficulty:  Hard        Goal:  7

109.  The manager of a restaurant measures several variables important to quality.  These include: food temperature, time to complete an order, and number of times a server visits a table.  These variables are graphed over time.  Is this an example of acceptance sampling or statistical process control?

Answer:  This is an example of statistical process control.  Several measures of process quality are measured over time.  Any unusual measurements will be detected in a timely fashion and the process can be corrected.        Difficulty: Hard       Goal:  5

110.  Explain the risks of acceptance sampling.

Answer:  There are two risks or acceptance sampling: consumer’s risk and producer’s risk.  In acceptance sampling, a decision to accept a batch of products is based on the inspection of a sample.  Based on the sample, if the sample of products is acceptable, then we make an inference that the entire batch is acceptable.  However, if this inference is incorrect, then this is consumer’s risk, i.e., we accept a batch that is not acceptable.  In contrast, if the sample of products is unacceptable, then we make an inference that the entire batch is unacceptable.  However, if this inference is incorrect, then this is producer’s risk, we rejected a batch that is acceptable.     Difficulty: Hard       Goal: 7