|  | Computer Engineering Department College of Computer and Information Sciences King Saud University | CEN 317: Logic Design Laboratory-II |
| --- | --- | --- |

Experiment 1: Introduction to ModelSim and Quartus II

Objectives

The objective of this Exercise is to learn how to use ActiveHDL to simulate a circuit, and how to use Quartus II to implement it on Altera DE1 Board. You will create a simple project to describe a 2X4 decoder. Then you will implement it on Altera DE1 Borad. Also, this exercise will help you to understand the way to create and instantiate components into your design.

Preparation

• Review the provided user manual for ActiveHDL, Quartus II, and tutorials for the Altera DE1 Board.

Equipment Needed

• ActiveHDL, Quartus II, and Altera DE1 Board

References

• User manual for Altera DE1 Board, Tutorials, and Exercises.

• Kevin Skahill, Verilog for Programmable Logic, Addison Wesley

• M. Morris Mano and Charles Kime, Logic and Computer Design Fundamentals, 2nd Edition, Prentice-Hall

Background

A Field-programmable Gate Array (FPGA) is an integrated circuit designed to be configured by the customer or designer after manufacturing—hence "field-programmable". The FPGA configuration is generally specified using a hardware description language (HDL).

HDL is commonly used to write text models that describe a logic circuit. Such a model is processed by a synthesis program, only if it is part of the logic design. A simulation program is used to test the logic design using simulation models to represent the logic circuits that interface to the design.

For this lab we will be using the Verilog to describe digital designs, ActiveHDL to simulate and verify our Verilog code, and Quartus II to implement the VHDL Designs on the Cyclone-II chip on your Altera DE1 Board.

The DE1 board provides 10 toggle switches, called SW9-0, that can be used as inputs to a circuit, and 10 red lights, called LEDR9-0, that can be used to display output values. The DE1 board has hardwired connections between its FPGA chip and the switches and lights. To use SW9-0 and LEDR9-0 it is necessary to include in your Quartus II project the correct pin assignments, which are given in the DE1 User Manual.

For this exercise, a simple example was chosen to get the requested familiarity. It is a 2x4 decoder as shown below:

| Inputs |  | Outputs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A | B | D0 | D1 | D2 | D3 |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 |

Prelab Assignments

1. Review the manual and tutorials for ActiveHDL.

2. Review the manual and tutorials for Quartus II.

3. Review user manual for the Altera DE1 Board.

4. Review how pin assignments are done.

Lab Assignments

For Part I (HomeWork):  Perform the following steps to Simulate and Test the 2x4 decoder:

- Write the Verilog code for 2x4 decoder.

- Simulate and verify your Verilog code using ActiveHDL.

For Part II(LabWork):  Perform the following steps to implement the 2x4 Decoder:

1. Create a new Quartus II project for your circuit.

2. Create a Verilog entity for the decoder. Connect its inputs to any two switches, and connect the four outputs to any four leds.

3. Include in your project the required pin assignments for the DE1 board. Compile the project.

4. Download the compiled circuit into the FPGA chip. Test the functionality of the decoder by toggling the two input switches and observing the LEDs.