|  | Computer Engineering Dept. College of Computer and Information Sciences King Saud University | CEN 317: Logic Design Laboratory-II |
| --- | --- | --- |

Experiment 2: Introduction to Altera DE1 Board

Objectives

The objective of this experiment is to gain an understanding of the Altera DE1 Board and create a simple project using input switches and the output LEDs. Also, to familiarize yourself with the way to create and instantiate components into your design

Preparation

• Review the provided user manual and tutorials for the Altera DE1 Board

Equipment Needed

• Altera DE1 Board

Background

The DE1 board provides 10 toggle switches, called SW9-0, that can be used as inputs to a circuit, and 10 red lights, called LEDR9-0, that can be used to display output values. The DE1 board has hardwired connections between its FPGA chip and the switches and lights. To use SW9-0 and LEDR9-0 it is necessary to include in your Quartus II project the correct pin assignments, which are given in the DE1 User Manual.

The purpose of this experiment is to learn how to connect simple input and output devices to an FPGA chip and implement a circuit that uses these devices. We will use the switches SW9-0 on the DE1 board as inputs to the circuit. We will use light emitting diodes (LEDs) and 7-segment displays as output devices.

Part I

Figure 1(a) shows a sum-of-products circuit that implements a 2-to-1 multiplexer with a select input s. If s = 0 the multiplexer’s output m is equal to the input x, and if s=1 the output is equal to y. Part (b) of the figure gives a truth table for this multiplexer, and part (c) shows its circuit symbol.

Figure 1: A 2-to-1 multiplexer.

The multiplexer can be described by the following Verilog statement:

m <= (NOT (s) AND x) OR (s AND y);

You are to write a VERILOG  entity that includes four assignment statements like the one shown above to describe the circuit given in Figure 2(a). This circuit has two four-bit inputs, X and Y, and produces the four-bit output M. If s = 0 then M=X, while if s=1 then M=Y. We refer to this circuit as a four-bit wide 2-to-1 multiplexer. It has the circuit symbol shown in Figure 2(b), in which X, Y, and M are depicted as four-bit wires.

Figure 2: A four-bit wide 2-to-1 multiplexer.

Part II

In Figure 1 we showed a 2-to-1 multiplexer that selects between the two inputs x and y. For this part consider a circuit in which the output m has to be selected from three inputs u, v, and w. Part (a) of Figure 3 shows how we can build the required 3-to-1 multiplexer by using two 2-to-1 multiplexers. The circuit uses a 2-bit select input s1s0 and implements the truth table shown in Figure 3(b). A circuit symbol for this multiplexer is given in part (c) of the figure. Recall from Figure 2 that a four-bit wide 2-to-1 multiplexer can be built by using four instances of a 2-to-1 multiplexer. Figure 4 applies this concept to define a two-bit wide 3-to-1 multiplexer. It contains two instances of the circuit in Figure 3(a).

Figure 3: A 3-to-1 multiplexer.

Figure 4: A two-bit wide 3-to-1 multiplexer.

Part III

Figure 5 shows a 7-segment decoder module that has the two-bit input c1c0. This decoder produces seven outputs that are used to display a character on a 7-segment display. Table 1 lists the characters that should be displayed for each valuation of c1c0. To keep the design simple, only three characters are included in the table (plus the ‘blank’ character, which is selected for codes 11).

The seven segments in the display are identified by the indices 0 to 6 shown in the figure. Each segment is illuminated by driving it to the logic value 0. You are to write a VERILOG  entity that implements logic functions that represent circuits needed to activate each of the seven segments. Use only simple VERILOG  assignment statements in your code to specify each logic function using a Boolean expression.

Figure 5: A 7-segment decoder.

Prelab Assignments

1. Review the manual and tutorials for the Altera DE1 Board.

2. Understand how to create and compile projects.

3. Review how pin assignments are done.

4. Review how to program the Altera DE1 Board.

Lab Assignments

For Part I:  Perform the following steps:

1. Create a new Quartus II project for your circuit.

2. Include your VERILOG  file for the four-bit wide 2-to-1 multiplexer in your project. Use switch SW9 on the DE1 board as the s input, switches SW3-0 as the X input and SW7-4 as the Y input. Connect the SW switches to the red lights LEDR and connect the output M to the green lights LEDG3-0.

3. Include in your project the required pin assignments for the DE1 board.

4. Compile the project.

5. Download the compiled circuit into the FPGA chip. Test the functionality of the four-bit wide 2-to-1 multiplexer by toggling the switches and observing the LEDs.

For Part II:  Perform the following steps to implement the two-bit wide 3-to-1 Mux:

1. Create a new Quartus II project for your circuit.

2. Create a VERILOG  entity for the two-bit wide 3-to-1 multiplexer. Connect its select inputs to switches SW9-8, and use switches SW5-0 to provide the three 2-bit inputs U to W. Connect the SW switches to the red lights LEDR and connect the output M to the green lights LEDG1-0.

3. Include in your project the required pin assignments for the DE1 board. Compile the project.

4. Download the compiled circuit into the FPGA chip. Test the functionality of the two-bit wide 3-to-1 multiplexer by toggling the switches and observing the LEDs. Ensure that each of the inputs U to W can be properly selected as the output M.

For Part III:  Perform the following steps:

1. Create a new Quartus II project for your circuit.

2. Create a VERILOG  entity for the 7-segment decoder. Connect the c1c0 inputs to switches SW1-0, and connect the outputs of the decoder to the HEX0 display on the DE1 board. The segments in this display are called HEX00, HEX01, ……, HEX06, corresponding to Figure 6. You should declare the 7-bit port

HEX0 : OUT STD_LOGIC_VECTOR(0 TO 6);

in your VERILOG  code so that the names of these outputs match the corresponding names in the DE1 User Manual and pin assignments.

3. After making the required DE1 board pin assignments, compile the project.

4. Download the compiled circuit into the FPGA chip. Test the functionality of the circuit by toggling the SW1-0 switches and observing the 7-segment display.