KSU: COLLEGE OF ENGINEERING

CIVIL ENGINEERING DEPARTMENT

CE417: CONSTRUCTION METHODES & EQUIPMENT

Exam I --  Year 1434 H

Prof  . Ibrahim Al Hammad

Student’s Full Name:__________________________Student’s ID Number_____________________

Q1 (25%)

A road surface has a dry earth with a rolling resistance factor of 66 kg/ton and coefficient of traction of 0.60.  If there is a wheel tractor-scraper whose weight on the driving wheels is 20000 kg and has a gross weight of 4000 kg, what is the maximum grade the scraper could ascend on the dry earth road.

Solution:

Given: type of road surface is dry earth, from table 4-2, we find the coefficient of traction is 0.60.

From equation 4-9, Maximum Usable Pull = 0.60   x   20000 kg = 12000 kg                 (1)

At maximum grade, the parrell load to the surface slope of the maximum slope of X% =

= 66 kg / ton   x  40000 / 1000 (ton) + X% x 40000 kg

= 2640 kg + 40000 kg . X%                       (2)

Equation 1 = Equation 2;  12000 kg = 2640 kg + 33000 kg. X%

9360 kg =  40000 kg. X%

Therefore  X%      =  0.234 = 23%

Q2 (25%)

Expalin and draw the principle method constructing facilities using a design/build contract?

Q3 (50%)

Elevations of the normal ground with respect to a proposed highway are as follows:

| Distance along centerline (ft) | 0 | 400 | 800 | 1200 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| Elevation (ft) | 0 | -100 | 0 | 120 | 0 | -88 |

Find the followings:

- Draw the highway profile

- Draw the mass diagram in CCY, if the conversion factor from Bank to Compacted volume is 0.90.

2. The total volume of a. cut; b. fill; c. waste OR borrow.

3. The average length of haul in the balanced sections.

| Highway Profile |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 |  |  |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |
| -40 |  | 200 |  | 400 |  |  |  |  |  |  |
| -80 |  |  |  |  |  |  |  |  |  |  |
| -120 |  |  |  |  |  |  |  |  |  |  |
| Mass Diagram |  |  |  |  |  |  |  |  |  |  |
|  |  | 200 |  | 400 |  |  |  |  |  |  |