Lab 3: SNMP in Windows

Based on NET311 – Computer Network Management

Instructor: Dr. Mostafa Dahshan

Objectives

- Install and configure SNMP service in Windows Server.

- Use a MIB browser software to interact with Windows SNMP agent.

References

- How to Install SNMP and Configure the Community String, Managed.com.

- SnmpB download, SourceForge.net.

Instructions

- Read the lab instructions.

- Provide question answers and screenshots in the supplied answer sheet.

- After finishing the lab, upload your saved answer sheet to LMS.

NET311	Dr. Mostafa Dahshan	Page 1 of 10

Part 1: Install and Configure SNMP Service in Windows Server

- Log in to Windows Server.

- Click on Windows Key > Administrative Tools > Server Manager.

- Click Manage > Add Roles and Features.

- Click Next > Next > Next > Next. Follow steps to install SNMP Service.

Lab sheet 1.1: provide a screenshot of the Add Roles and Features screen.

NET311	Dr. Mostafa Dahshan	Page 2 of 10

- Click on Windows Key > Administrative Tools > Services.

- Right click on SNMP Service and click on Properties.

- Click on the Security tab.

- Type your Community Name. Make sure it is READ ONLY.

Lab sheet 1.2: type the value of the Community Name.

Lab sheet 1.3: provide a screenshot of your Security configuration screen.

NET311	Dr. Mostafa Dahshan	Page 3 of 10

9. Click on the Agent tab and enter your Contact and Location information.

Lab sheet 1.4: type the contact information you entered.

Lab sheet 1.5: provide a screenshot of your Agent configuration screen.

NET311	Dr. Mostafa Dahshan	Page 4 of 10

Part 2: Use a MIB browser to interact with Windows SNMP agent

- Install the SnmpB software.

- Run the SnmpB software.

NET311	Dr. Mostafa Dahshan	Page 5 of 10

- Click Options > Manage Agent Profiles.

- Expand the localhost profile.

- Under Snmpv1/v2c, enter the Read Community value configured in Part 1.

6. Click OK.

Lab sheet 2.1: provide a screenshot of the running SnmpB screen.

NET311	Dr. Mostafa Dahshan	Page 6 of 10

7. Expand the MIB Tree > iso > org > dod > internet > mgmt > mib-2 > system

Lab sheet 2.2: type the value of the OID you selected.

NET311	Dr. Mostafa Dahshan	Page 7 of 10

8. Right-click on system, then click on Walk.

NET311	Dr. Mostafa Dahshan	Page 8 of 10

Lab sheet 2.3: provide the Query Result contents (copy/paste).

NET311	Dr. Mostafa Dahshan	Page 9 of 10

Extra Tasks

Load more modules in SnmpB that can help finding more OIDs. For example, use IF-MIB to find the ifDescr.

Use alternative SNMP browser tool, such as snmputilg.

NET311	Dr. Mostafa Dahshan	Page 10 of 10