# Lab 8: Basic Inter-VLAN Routing

## Topology Diagram

## Addressing Table

| Device (Hostname) | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 | 172.17.99.1 |
| S2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 | 172.17.99.1 |
| S3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 | 172.17.99.1 |
| R1 | Fa 0/0 | 172.17.50.1 | 255.255.255.0 | N/A |
| R1 | Fa 0/1 | See Interface Configuration Table |  | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.1 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.1 |
| Server | NIC | 172.17.50.254 | 255.255.255.0 | 172.17.50.1 |

## Port Assignments – Switch 2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.17.99.0 /24 |
| Fa0/6 – 0/10 | VLAN 30 – Guest (Default) | 172.17.30.0 /24 |
| Fa0/11 – 0/17 | VLAN 10 – Faculty/Staff | 172.17.10.0 /24 |
| Fa0/18 – 0/24 | VLAN 20 - Students | 172.17.20.0 /24 |

## Interface Configuration Table – Router 1

| Interface | Assignment | IP Address |
| --- | --- | --- |
| Fa0/1.1 | VLAN1 | 172.17.1.1 /24 |
| Fa0/1.10 | VLAN 10 | 172.17.10.1 /24 |
| Fa0/1.20 | VLAN 20 | 172.17.20.1 /24 |
| Fa0/1.30 | VLAN 30 | 172.17.30.1 /24 |
| Fa0/1.99 | VLAN 99 | 172.17.99.1 /24 |

## Learning Objectives

- Configuration of a switched LAN and router

- VLANs and VLAN Trunking Protocol (VTP)

- Router and 802.1q trunking on a Fast Ethernet interface

- Subinterfaces corresponding to the configured VLANs

- Inter-VLAN routing

Task 1: Prepare the Network

Step 1: Cable a network that is similar to the one in the topology diagram.

The output shown in this lab is based on 2960 switches and an 1841 router.

Ethernet (10Mb) LAN interfaces on routers do not support trunking, and Cisco IOS software earlier than version 12.3 may not support trunking on Fast Ethernet router interfaces.

Step 2: Clear existing configurations on the switches.

Clear NVRAM, delete the vlan.dat file, and reload the switches. After the reload is complete, use the show vlan command to confirm that only default VLANs exist and that all ports are assigned to VLAN 1.

Switch#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15,Fa0/16

Fa0/17, Fa0/18, Fa0/19,Fa0/20

Fa0/21, Fa0/22, Fa0/23,Fa0/24

Gig0/1, Gig0/2

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

Step 3: Disable all ports using the shutdown command.

Use the interface range command. Repeat these commands on each switch in the topology.

Switch(config)#interface range fa0/1-24

Switch(config-if-range)#shutdown

Switch(config-if-range)#interface range gi0/1-2

Switch(config-if-range)#shutdown

Task 2: Perform Basic Switch Configurations

Step 1: Configure the S1, S2, and S3 switches.

Use the addressing table and the following guidelines:

- Configure the switch hostname.

- Disable DNS lookup.

- Configure an enable secret password of class.

- Configure a password of cisco for console connections.

- Configure a password of cisco for vty connections.

- Configure the default gateway on each switch

Output for S1 shown

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname S1

S1(config)#enable secret class

S1(config)#no ip domain-lookup

S1(config)#ip default-gateway 172.17.99.1

S1(config)#line console 0

S1(config-line)#password cisco

S1(config-line)#login

S1(config-line)#line vty 0 15

S1(config-line)#password cisco

S1(config-line)#login

S1(config-line)#end

%SYS-5-CONFIG_I: Configured from console by console

S1#copy running-config startup-config

Destination filename [startup-config]? [enter]

Building configuration...

Step 2: Re-enable the active user ports on S2 in access mode.

S2(config)#interface fa0/6

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

S2(config-if)#interface fa0/11

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

S2(config-if)#interface fa0/18

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

Task 3: Configure the Ethernet Interfaces on the Host PCs

Configure the Ethernet interfaces of PC1, PC2, PC3 and the remote TFTP/Web Server with the IP addresses from the addressing table.

Task 4: Configure VTP on the Switches

Step 1:  Configure VTP on the three switches using the following table. Remember that VTP domain names and passwords are case-sensitive.

| Switch Name | VTP Operating Mode | VTP Domain | VTP Password |
| --- | --- | --- | --- |
| S1 | Server | Lab6 | cisco |
| S2 | Client | Lab6 | cisco |
| S3 | Client | Lab6 | cisco |

S1:

S1(config)#vtp mode server

Device mode already VTP SERVER.

S1(config)#vtp domain Lab6

Changing VTP domain name from NULL to Lab6

S1(config)#vtp password cisco

Setting device VLAN database password to cisco

S1(config)#end

S2:

S2(config)#vtp mode client

................................

S3:

S3(config)#vtp mode client

................................

Step 2: Configure trunking ports and designate the native VLAN for the trunks.

Configure Fa0/1 through Fa0/5 as trunking ports, and designate VLAN 99 as the native VLAN for these trunks. Use the interface range command.

S1(config)#interface range fa0/1 - 5

S1(config-if-range)#switchport mode trunk

S1(config-if-range)#switchport trunk native vlan 99

S1(config-if-range)#no shutdown

S1(config-if-range)#end

S2(config)# interface range fa0/1 - 5

......................................................................

S3(config)# interface range fa0/1 - 5

...........................................................................

Step 3: Configure VLANs on the VTP server.

Configure the following VLANS on the VTP server:

| VLAN | VLAN Name |
| --- | --- |
| VLAN 99 | management |
| VLAN 10 | staff |
| VLAN 20 | students |
| VLAN 30 | guest |

S1(config)#vlan 99

S1(config-vlan)#name management

S1(config-vlan)#exit

S1(config)#vlan 10

S1(config-vlan)#name staff

S1(config-vlan)#exit

S1(config)#vlan 20

S1(config-vlan)#name students

S1(config-vlan)#exit

S1(config)#vlan 30

S1(config-vlan)#name guest

S1(config-vlan)#end

Verify that the VLANs have been created on S1 with the show vlan brief command.

Step 4: Verify that the VLANs created on S1 have been distributed to S2 and S3.

Use the show vlan brief command on S2 and S3 to verify that the four VLANs have been distributed to the client switches.

S2#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12,Fa0/13

Fa0/14, Fa0/15, Fa0/16,Fa0/17

Fa0/18, Fa0/19, Fa0/20,Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gi0/1

Gi0/2

10   staff                    active

20   students                         active

30   guest                            active

99   management                       active

Step 5: Configure the management interface address on all three switches.

S1(config)#interface vlan 99

S1(config-if)#ip address 172.17.99.11 255.255.255.0

S1(config-if)#end

S2(config)#interface vlan 99

S2(config-if)#ip address 172.17.99.12 255.255.255.0

S2(config-if)#end

S3(config)#interface vlan 99

S3(config-if)#ip address 172.17.99.13 255.255.255.0

S3(config-if)#end

Verify that the switches are correctly configured by pinging between them. From S1, ping the management interface on S2 and S3. From S2, ping the management interface on S3.

Were the pings successful?

All pings should be successful.

If not, troubleshoot the switch configurations and try again.

Step 6: Assign switch ports to VLANs on S2.

Refer to the port assignments table at the beginning of the lab to assign ports to VLANs on S2.

S2(config)#interface range fa0/6-10

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 30

S2(config-if-range)#interface range fa0/11-17

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 10

S2(config-if-range)#interface range fa0/18-24

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 20

S2(config-if-range)#end

S2#copy running-config startup-config

Destination filename [startup-config]? [enter]

Building configuration...

[OK]

Step 7: Check connectivity between VLANs.

Ping from PC1 (172.17.10.21) to PC2 (172.17.20.22). Ping from PC2 to PC3 (172.17.30.23).

Are the pings successful?

These pings are not successful.

If not, why do these pings fail?

Each host is in a different VLAN. Because each VLAN is in a separate Layer 3 domain, packets need to be routed at Layer 3 between VLANs. We have not yet configured the devices with L3 capability.

Task 5: Configure the Router and the Remote Server LAN

Step 1: Clear the configuration on the router and reload.

Router#erase nvram:

Erasing the nvram filesystem will remove all configuration files! Continue? [confirm]

Erase of nvram: complete

Router#reload

System configuration has been modified. Save? [yes/no]: no

Step 2: Create a basic configuration on the router.

- Configure the router with hostname R1.

- Disable DNS lookup.

- Configure an EXEC mode password of cisco.

- Configure a password of cisco for console connections.

- Configure a password of cisco for vty connections.

Step 3: Configure the trunking interface on R1.

- Enter subinterface configuration mode

- Establish trunking encapsulation

- Associate a VLAN with the subinterface

- Assign an IP address from the VLAN to the subinterface

R1(config)#interface fastethernet 0/1 
R1(config-if)#no shutdown

R1(config-if)#interface fastethernet 0/1.1 
R1(config-subif)#encapsulation dot1q 1 
R1(config-subif)#ip address 172.17.1.1 255.255.255.0

R1(config-if)#interface fastethernet 0/1.10
R1(config-subif)#encapsulation dot1q 10 
R1(config-subif)#ip address 172.17.10.1 255.255.255.0

R1(config-if)#interface fastethernet 0/1.20
R1(config-subif)#encapsulation dot1q 20 
R1(config-subif)#ip address 172.17.20.1 255.255.255.0

R1(config-if)#interface fastethernet 0/1.30
R1(config-subif)#encapsulation dot1q 30 
R1(config-subif)#ip address 172.17.30.1 255.255.255.0

R1(config-if)#interface fastethernet 0/1.99
R1(config-subif)#encapsulation dot1q 99 native
R1(config-subif)#ip address 172.17.99.1 255.255.255.0

- Router interfaces are down by default. The virtual interfaces are up by default. The subinterface can use any number, but it is good practice to assign the number of the VLAN as the interface number.

- The native VLAN is specified on the L3 (Layer 3) device ( = router) so that it is consistent with the switches. Otherwise, VLAN 1 would be the native VLAN by default, and there would be no communication between the router and the management VLAN on the switches.

Confirm creation and status of the subinterfaces with the show ip interface brief command:

R1#show ip interface brief

Interface           IP-Address      OK? Method Status                Protocol

FastEthernet0/0     unassigned      YES unset  administratively down down

FastEthernet0/1     unassigned      YES unset  up                    up

FastEthernet0/1.1   172.17.1.1      YES manual up                    up

FastEthernet0/1.10  172.17.10.1     YES manual up                    up

FastEthernet0/1.20  172.17.20.1     YES manual up                    up

FastEthernet0/1.30  172.17.30.1     YES manual up                    up

FastEthernet0/1.99  172.17.99.1     YES manual up                    up

Step 4: Configure the server LAN interface on R1.

R1(config)# interface FastEthernet0/0

R1(config-if)#ip address 172.17.50.1 255.255.255.0

R1(config-if)#description server interface

R1(config-if)#no shutdown

R1(config-if)#end

There are now six networks configured. Verify that you can route packets to all six by checking the routing table on R1.

R1#show ip route

<output omitted>

Gateway of last resort is not set

172.17.0.0/24 is subnetted, 6 subnets

C       172.17.50.0 is directly connected, FastEthernet0/0

C       172.17.30.0 is directly connected, FastEthernet0/1.30

C       172.17.20.0 is directly connected, FastEthernet0/1.20

C       172.17.10.0 is directly connected, FastEthernet0/1.10

C       172.17.1.0 is directly connected, FastEthernet0/1.1

C       172.17.99.0 is directly connected, FastEthernet0/1.99

If your routing table does not show all six networks, troubleshoot your configuration.

Step 5: Verify Inter-VLAN routing.

From PC1, verify that you can ping the remote server (172.17.50.254) and the other two hosts (172.17.20.22 and 172.17.30.23). It may take a couple of pings before the end-to-end path is established.

Are the pings successful?

These pings should be successful.

If not, troubleshoot your configuration.

Task 6: Reflection

In Task 5, it was recommended that you configure VLAN 99 as the native VLAN in the router Fa0/0.99 interface configuration. Why would packets from the router or hosts fail when trying to reach the switch management interfaces if the native VLAN were left in default?

The native VLAN is untagged. If the VLAN 99 traffic to the router is untagged (as it would be because that is native on the switches), the router cannot interpret the data because there is no VLAN information in the header as expected. In turn, the router tags all VLAN 99 traffic outbound, and leaves VLAN 1 data untagged, so the switches are unable to correctly interpret either. VLAN traffic to the other VLANs should not be affected by the assignment of the native VLAN.

Final Configurations

Router 1

hostname R1

!

enable secret class

!

no ip domain lookup

!

interface FastEthernet0/0

ip address 172.17.50.1 255.255.255.0

no shutdown

!

interface FastEthernet0/1

no shutdown

!

interface FastEthernet0/1.1

encapsulation dot1Q 1

ip address 172.17.1.1 255.255.255.0

!

interface FastEthernet0/1.10

encapsulation dot1Q 10

ip address 172.17.10.1 255.255.255.0

!

interface FastEthernet0/1.20

encapsulation dot1Q 20

ip address 172.17.20.1 255.255.255.0

!

interface FastEthernet0/1.30

encapsulation dot1Q 30

ip address 172.17.30.1 255.255.255.0

!

interface FastEthernet0/1.99

encapsulation dot1Q 99 native

ip address 172.17.99.1 255.255.255.0

!

<output omitted - serial interfaces not configured>

!

line con 0

line aux 0

line vty 0 4

login

password cisco

!

Switch 1

!

hostname S1

!

enable secret class

!

no ip domain lookup

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

<output omitted - all remaining ports in shutdown>

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan99

ip address 172.17.99.11 255.255.255.0

no shutdown

!

ip default-gateway 172.17.99.1

ip http server

!

line con 0

logging synchronous

line vty 0 4

login

password cisco

line vty 5 15

login

password cisco

Switch 2

!

hostname S2

!

enable secret class

!

no ip domain lookup

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/6

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 30

!

interface FastEthernet0/8

switchport access vlan 30

!

interface FastEthernet0/9

switchport access vlan 30

!

interface FastEthernet0/10

switchport access vlan 30

!

interface FastEthernet0/11

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 10

!

interface FastEthernet0/13

switchport access vlan 10

!

interface FastEthernet0/14

switchport access vlan 10

!

interface FastEthernet0/15

switchport access vlan 10

!

interface FastEthernet0/16

switchport access vlan 10

!

interface FastEthernet0/17

switchport access vlan 10

!

interface FastEthernet0/18

switchport access vlan 20

!

interface FastEthernet0/19

switchport access vlan 20

!

interface FastEthernet0/20

switchport access vlan 20

!

interface FastEthernet0/21

switchport access vlan 20

!

interface FastEthernet0/22

switchport access vlan 20

!

interface FastEthernet0/23

switchport access vlan 20

!

interface FastEthernet0/24

switchport access vlan 20

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan99

ip address 172.17.99.12 255.255.255.0

no shutdown

!

ip default-gateway 172.17.99.1

ip http server

!

line con 0

password cisco

logging synchronous

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

!

end

Switch 3

!

hostname S3

!

enable secret class

!

no ip domain lookup

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

<output omitted - all remaining ports in shutdown>

!

interface Vlan99

ip address 172.17.99.13 255.255.255.0

no shutdown

!

ip default-gateway 172.17.99.1

ip http server

!

control-plane

!

line con 0

password cisco

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

!

end