Experiment - SNMP

The medway-ME router is configured to serve an SNMP community medway-public for read-only access (snmp-server community medway-public ro ), and

| Device | Addresses | Passwords |
| --- | --- | --- |
| PC-PT | IP address: 10.0.0.2 Subnet Mask: /24 Default Gateway: 10.0.0.1 |  |
| Router (Medway-ME) | FastEithernet 0/0:10.0.0.1 Subnet Mask: /24 FastEithernet 0/1:10.0.1.1 Subnet Mask: /24 | Read-Only: medway-public Read-Write: medway-private |
| Server-PT | IP address: 10.0.2.2 Subnet Mask: /24 Default Gateway: 10.0.0.1 |  |
| Router (Nelson-south) | FastEithernet 0/0: 10.0.1.2 Subnet Mask: /24 FastEithernet 0/1: 10.0.2.1 Subnet Mask: /24 | Read-Only: nelson-public Read-Write: nelson-private |

Test SNMP Requests from medway-ME router

- Prepare the devices with the addresses.

- Prepare the routers with the snmp passwords.

Medway-ME (config)#snmp-server community public RO
	Medway-ME (config)#snmp-server community private RW

- Open  the MIB browser of the PC-PT and access Router(Medway-ME) information:

MIB Tree.router_std MIBs.iso.org.dod.internet.mgmt.mib-2.system.sysDescr

OID=…………………

- Ensure that Get is selected in the Operations field of the MIB browser. Enter the Router IP in the address field in the left handside, Press the GO button to retrieve the value from the device.

- Now select the node with the name:

MIB Tree.router_std MIBs.iso.org.dod.internet.mgmt.mib-2.system.sysUpTime

OID=…………………..

- Ensure that Get is selected in the Operations field of the MIB browser. Press the GO button to retrieve the value from the device.

How long has the router been 'up'?......................

Setting Variables

- Select the .sysContact node and change the operation to Set.

Select the OctetString data option and enter your name.

Press GO.

- Change the operation back to Get and press GO again to verify that the data was set correctly.

- Set the .sysLocation node value to ME4 4TB and verify that it has been set correctly.

NOTE: SET operation could be used only with variables that have (read-Write) access

Using Get Bulk

- Change the SNMP version in the advances button of the MIB browser to v3, then expand the MIB tree on the left side of the application to select the node with the name:

MIB Tree.router_std MIBs.iso.org.dod.internet.mgmt.mib-2.interfaces

Ensure that Get Bulk is selected in the Operations field of the MIB browser. Press the GO button to retrieve the values from the device.

NOTE: GET BULK works only with snmp version 3

You can repeat the steps from 3 to 10 to access the router (Nelson South) from the Server-PT