| - قسم الهندسة الميكانيكية | - ME 253 MATERIALS ENGINEERING - Fall 2016, 1438 - 1nd Mid-term exam - 30/10/2016 (29/1/1438) | - Grade |  |
| --- | --- | --- | --- |
|  |  | - 1st |  |
|  |  | - 2nd |  |
|  |  | - 3rd |  |
|  |  | - 4th |  |
|  |  | - Total - /40 |  |

Name:						No.                               Group:

Given data:  NA= 6.023x1023 atoms/mol.

| Problem 1 (15 points) CLO 1 - True (T) or False (F) (5 points) - Materials Science is mainly concerned with the use of various materials in different engineering applications. - Materials are basically classified according to atomic bonding. - Materials is classified into four groups; metals, polymers, ceramics and composites based on chemical bonding. - The number of atoms in FCC unit cell is 4. - [111] direction in cubic system is perpendicular to (111−) plane - Metallic elements readily lose valence electrons, while nonmetallic elements accept or share electrons. - Ceramic materials are electrical insulator because they are hard and brittle. - Metallic bond is characterized by high directionality and based on valence electrons. - Crystalline materials are usually poly-crystalline materials in nature. - Primary atomic bonding is weaker than hydrogen bonding. | - Ag has an atomic weight 107.87 gm/mol and density of 10.49 g/cm3. Calculate number of atoms in mm3. - Be (1s2 2s2) and O (1s22s22p4) suggest the type of bonding and draw the molecule. |
| --- | --- |

| Problem 2 (16points) - Within a cubic unit cell, sketch the following direction and plane. 310, 120 b) What are the Miller Indices for the following directions and plane in the following unit cells. | c) The [102] direction that passes through the point 12,12,12, passes through the point, --, ---, --. d) Find the linear density LD111 and planar density PD110 in BCC unit cell. |
| --- | --- |

| Problem 3 (14 points) - Titanium (Ti) has HCP unit cell and atomic packing factor of 0.74. Find the height c of unit cell. Given that the atomic radius is 0.145 nm and the c/a ratio is 1.633. | - For the Hexagonal unit cell draw: 011−0, 11−01, <21−1−0> |
| --- | --- |