Name in Arabic :							      Lecture time :

Number  in Class:

KING SAUD UNIVERSITY				                       STEEL STRUCTURES : CE 473

COLLEGE OF ENGINEERING					            FIRST  SEMESTER, 1428/1429 H

CIVIL ENGINEERING DEPARTMENT					TIME : 90 min

SECOND MID TERM EXAM

Answer all problems in the provided spaces

The shown steel structure is laterally supported at points D, E, F, G, H, and I. The steel sections for columns and beams are as shown and are made of steel A36.  The braces AE and BD are double angles back to back and connected at point of intersection. All columns are fixed at base.

Column CFI is a built up w-shape made of 2 plates 400 x10 mm as flanges and 500 x 8 mm as web.

G

H

I

W 360 x 134

W 460 x 158

W 310 x 143

W 310 x 143

3 m

D

E

W 150 x 29.8

F

4 m

W 310 x 143

A

B

C

x

y

x

y

6 m

10 m

- What are the buckling lengths of member AE?

|  | Choose the right answer |  | value |
| --- | --- | --- | --- |
|  | Buckling in plane | Buckling outside plane |  |
| KLx |  |  |  |
| KLy |  |  |  |

  - Choose the right answer for columns AD and  CFI  in the table below

|  |  | Buckling in plane | Buckling outside plane |
| --- | --- | --- | --- |
| Column AD | KLx |  |  |
|  | KLy |  |  |
| Column CFI | KLx |  |  |
|  | KLy |  |  |

  - If member AE has a buckling length of KLx = 3.0 m and  KLy = 6.0 m, and a factored compression force of  300 kN,  Use the LRFD tables

      - Find the lightest double unequal angles with the short leg back to back

b- Determine its factored compression resistance

  - Determine the buckling lengths KLx and KLy for column CFI,

given that for column CFI;   Ix = 603 x10 6 mm4 ,     Iy= 106 x10 6 mm4

  - If the section of beam HI is W 460 x 158 and its both flanges are laterally supported at ends, with the given ultimate bending moment diagram, check the safety of the beam.

300 kN.m

450 kN.m

680 kN.m

300 kN.m

150 kN.m

2.5 m

2.5 m

2.5 m

2.5 m

6-  If  the Column CFI is a built up w-shape made of 2 plates 400 x10 mm as flanges and 500 x 8 mm as web, and the buckling length of member CFI  are KLx = 11 m and KLy = 3.50 m, given that ;   Ix = 603 x10 6 mm4 , Iy= 106 x10 6 mm4 and A = 12000 mm2. Determine its factored compression resistance.