Solution of 2nd  Midterm Exam ( 2nd semester 28/29, 5/5/29(10/5/08))

Q1  A)

For  permeate membrane , Transfer in the membrane occur  by diffusion

Fick Law apply : NA= - D (dCA/dx )

Integration under constant area :

NA = D* [ (CA1-CA2) / ( x2-x1 ) ]

Substitute CA1= ( S * PA) / 22.41

NA= D*S [ ( PA1-PA2 ) / L * 22.414 ]

NA1= PM1 /22.414 [ (PA1-PA2)/ L1  --1

NA2= PM2 /22.414 [ (PA2-PA3)/ L2  --2

Rearrange eq.1&2 in the following form

PA1-PA2= NA1 * 22.414( L1/PM1)  --3

PA2-PA3= NA2 * 22.414( L2/PM2)  --4

-------------------------------------------------

-  add eq.3&4

- At steady state , series membrane  NA1=NA2 =NA

PA1-PA3= NA* 22.414 [ L1/PM1 + L2/PM2 ]

NA = ( PA1-PA3 ) / [ 22.414 (L1/PM1 + L2/PM2 ) ]

-------------------------------------------------------------------------------------------------------

Q1 B)

- Three region of mass transfer occur it can be distinguished by Knudsen Number ( N kn) = λ /d   ( Bulk diffusion  Nkn < 0.01 , Knudsen diffusion  Nkn> 10 ,  Transition diffusion       0.01  <Nkn>  10 )

- - Equimolar counter diffusion  α = 0  ,  using X avg = (Xa1+Xa2) / 2

- Bulk diffusion  D α 1/p  ;    Knudsen diffusion  D is independent of pressure

- Jd = Nst ( Nsc )0.67  -- 1   , Nst = Nsh/ NRe*Nsc  -----2      from 2 into 1                               Jd = NSh * ( Nsc )0.67  /  NRe*Nsc  = Nsh / NRe * Nsc0.33

Q1 c)

- Film model : The resistance to mass transfer occur in a thin film near the wall were transfer occur by diffusion , beyond this film mixing is high that keep concentration constant

Na= D / ∆ x  ( Ca1- Ca2 )

Na = K  ( Ca1- Ca2 )

K  = D / ∆ x

Parameter affecting K

- Velocity of fluid

- Properties of fluid

- Configuration

- 1- Reynold analogy apply for gas only with skin friction

f/2= (h/ ρ * v * Cp ) = ( k’c / v )

Chilton coluburn analogy

Jd = Jh = f / 2      [ Jh = NPr * (h/ ρ * v * Cp ) ] , Jd = Nst ( Nsc )0.67

Apply for gas and liquid ,with skin friction , for drag friction Jd=Jh

2- Direct processes involve processes were heat is added or removed or

removed and added ( e.g. distillation , stripping, crystallization,…)

Indirect processes involve processes that use forign substance

( e.g. absorption , leaching, Ion exchange,….)

Q2     NRe          ρ v d / μ = 0.0695 * 3600 *0.1 / 0.0467 = 535.8

Nsc           μ / ρ * D= 0.0467/ 0.0695 * 0.2682         = 2.505

Nsh = 2 +  0.552 NRe0.52 Nsc 0.33 = kc’ * d / D =  21.62

K’c = 21.62* 0.2682 / 0.1= 58 ft/sec

K’c =Kc  ( Ppm = 1 )

  - Nst =Kc / v = NSh / NRe * NSc = 58 / 3600 = 0.0161

  - Jd = Nst ( NSc ) 0.67 = 0.0161 * 1.85  = 0.03

  - Flux = Kc ( CA1-CA2) = Kc/R*T (PA1-PA2)= Kg ( PA1-PA2)

PA1 = 0.555/760  , PA2= 0.0 , T = 113+ 460= 573 R

Flux = ( 58/ 0.73 * 573 ) * ( 0.555/760 – 0) = 1.013x 10-4  lbmole/hr ft2

Rate = Flux *Area

Area = 4πr2    =  πD 2 = 0.0314 ft2

Rate = 1.013x 10-4 * 0.0314 = 3.181x 10-6 lb mole / hr

  - V=0,K=2,  K = 2 * 0.2682 / 0.1 =  5.364 ft/ hr ( natural conv.)

  - ∆ x = D / Kc = 0.2682 / 58 = 4.62x10-3 ft